|  |  | MANIPAL INSTITUTE OF TECHNOLOGY (A Constituent Institute of MANIPAL UNIVERSITY) Department of Electrical And Electronics Engineering MANIPAL - 576 104, Karnataka, India. |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Subject: |  | ELE1001: BASIC ELECTRICAL TECHNOLOGY [2 1 0 3] |  |  |  |
| - Find the RMS value of the half-wave rectified sinusoidal waveform shown in Fig Q.1. - An e.m.f. of 10 sin (314t + 50) volts is applied to an inductive circuit and the current in the circuit is 2 sin (314t – 30) amperes. Determine, - Frequency - Resistance and inductance of the circuit - Active power absorbed - If a series capacitance is added so as to bring the circuit into resonance at this frequency, find the value of capacitor and the expression for the current. - For the circuit shown in Fig Q.3, find the value of C such that the total current will be in phase with the source voltage. Also, find the current drawn and the power absorbed. - A coil of resistance 50 Ω and inductance 0.318 H is connected in parallel with a circuit comprising a 75 Ω resistor in series with a 159 μF capacitor. The resulting circuit is connected to a 230V, 50Hz a c supply. Calculate: - The supply current - The circuit impedance, resistance and reactance - A coil takes a current of 10 A and dissipates 1410 W when connected to a 230 V, 50 Hz sinusoidal supply. When another coil is connected in parallel with it, the total current taken from the supply is 20 A at a power factor of 0.866. Determine the current and the overall power factor when the coils are connected in series across the same supply. - For the Fig. Q.6 find the value of R and C so that Vb = 3Va and Vb and Va are in quadrature (i.e. 90°out of phase). - A current of 5 A flows through a non-inductive resistance in series with a coil, as shown in Fig. Q.7, when supplied at 250 V, 50 Hz. If the voltage across the resistance is 125 V and that across the coil is 200V, calculate - The non-inductive resistance value, - Impedance, reactance and resistance of the coil - Power absorbed by the coil - Total power consumed by the circuit - Power factor of the circuit - Draw the Phasor diagram - Use node voltage method to obtain the current through the inductor in the network shown in Fig. Q.8. - A house has following loads connected to 230 V, 50 Hz supply. Calculate the value of capacitor bank to be connected in parallel to improve the power factor to 0.95 lag Load; Rating; Quantity. Incandescent bulb; 60W; 3. Tube light; 40W, pf = 0.85; 2. Fan; 70W, pf = 0.75; 2. TV; 150W, pf = 0.7; 1. - A resistor and a capacitor are in series with a variable inductor. When the circuit is connected to a 200 V, 50 Hz supply, the maximum current obtainable by varying the inductance is 0.314 A. The voltage across the capacitor is then 300 V. Find the circuit constants |
| Answers: |  |  |  |  |  |
| 1. | 5 |  | 6. | R= 2.66 Ω, C=1.59 mF |  |
| 2. | Z = (0.868 + j 4.924 ) Ω, P = 1.74 W, C = 0.647 mF |  | 7. | R = 25 Ω, Zcoil = 40 Ω, RL = 5.5 Ω, XL = 39.62 Ω, Pcoil= 137.5 W, Total Power= 762.5W, Total pf= 0.61 |  |
| 3. | C= 318.31µF |  | 8. | I = 0.135∠2.6 A |  |
| 4. | I = 3.9∠-15.81 A, Z= (56 + j 16) Ω |  | 9. | C= 8.742 µF |  |
| 5. | I= 5.34 A, pf= 0.857 (lagging) |  | 10. | C= 3.3 µF, L= 3.04 H, R= 637 Ω |  |